| Type / Built | Self-trimming Double Hull Post Panamax Bulk Carrier / 2005 |
| --- | --- |
| Yard | IHI Marine United Inc, Yokohama |
| Flag / Class / Callsign / IMO | Singapore / Lloyd’s / 9VBK4 / 9296858 |
| Dwt / Dft ( Summer) | 87,052 mt on 14.14 m ssw |
| LOA / Beam / TPC | 229 m / 36.50 m / 77.77 mt |
| GRT / NRT | 46,982 / 26,950 |
| Holds / Hatches /Hatch type | 7 HO / 7 HA / Side rolling and hydraulic chain driven type |
| Hatch sizes ( LxB ) | 1: 13.65 X 12.70 m |
|  | 2-7: 17.10 X 16.80 m |
| Cargo gears | Gearless |
| Grain / Bale | 98,961.52 Cbm / 94,844.04 Cbm |
| Hold capacity breakdown | 1 : 10,771.30 m3 / 10,305.47 m3 |
|  | 2 : 14,852.03 m3 / 14,236.04 m3 |
|  | 3 : 14,919.87 m3 / 14,283.24 m3 |
|  | 4 : 14,936.67 m3 / 14,283.20 m3 |
|  | 5 : 14,966.98 m3 / 14,329.73 m3 |
|  | 6 : 14,782.40 m3 / 14,174.04 m3 |
|  | 7 : 13,732.27 m3 / 13,232.32 m3 |
| Tank Top Strength | 32.10 mt/m2 |
|  | Speed / Consumption in good weather conditions up to BF4 and DSS 3 and no negative swell / no adverse current: (IFO 380 CST ISO 8217 RMG35 / MDO ISO 8217 DMB) |
| Speed / consumption ( Ballast ) | Abt 14k on abt 39.5 mt IFO + abt 0.2 mt MDO |
| Speed / consumption ( Loaded ) | Abt 13.5k on abt 39.5 mt IFO + abt 0.2 mt MDO |
| Port Consumption | Working: Abt 2.8 mt MDO |
|  | Idle: Abt 2.6 mt MDO |
|  | Vessel entitles to use more diesel oil at narrow/shallow/busy water area and engine starting/stopping. |
| Owners | Alam Pintar Maritime Ltd, UK |
| Managers | Paccship (UK) Ltd |
| AHL / Alternate holds load | Yes, No. 2, 4 & 6 holds may be empty. |
| (ALL DTLS ABT WOG) |  |